Lab_2: Add and Shift Multiplier

Tip 2: system block diagram

System block diagram

This diagram is not really a system block diagram, as it does not show all the ports of each module, but it does show the general structure of the system and how the modules communicate with each other.

If you see this and you think you know exactly how to write the code, you can close this document. If not, I will provide more instructions below.

Adder

It’s just a simple combinatorial logic component. All you have to do is add the two input numbers and output the result.

Register

Remember the counter that was done in the last experiment? If we replace the count function with the shift function and the other functions remain the same, we get an important part of this system. In fact, components M, A and Q are all register.

From the connection of the control signal, we can see that A needs to use the “Load”, “Shift” and “Clear” functions, Q does not need to use the ”Clear” function, and M only needs to use the “Load” function.

Controller

In fact, you know how the control signal from the FSM output should be connected to the Register, right? For example, if you want to perform an “Add” operation, you should set the Load signal of A to 1 so that A will save the sum.