Student Name Chinese:    	       _____________________
Student Name Pinyin:    		_____________________
Student Email:    			_____________________
Student Id:  				_____________________
School:           			_____________________
Year of Entrance:  			_____________________

ShanghaiTech University Final Examination Cover Sheet

Jan 4 2026

Academic Year:         	2025 - 2026

Term:  			Fall

Course-offering School:  	School of Information Science and Technology

Instructor:           		Yajun Ha

Course Name:        		Reconfigurable Computing

Course Number: 	     	EE216

Exam Instructions for Students:
1. All examination rules must be strictly observed throughout the entire test, and any form of cheating is prohibited.

2. Other than allowable materials, students taking closed-book tests must place their books, notes, tablets and any other electronic devices in places designated by the examiners.

3. Students taking open-book tests may use allowable materials authorized by the examiners. They must complete the exam independently without discussion with each other or exchange of materials.

For Marker’s Use:

| Section | Q.1 | Q.2 | Q.3 | Q.4 | Q.5 | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Max | 25 | 20 | 13 | 12 | 30 | 100 |
| Marks |  |  |  |  |  |  |
| Recheck |  |  |  |  |  |  |

Q.1

(a)Which of the following statements about initial and always is incorrect? _________. (4 marks)

A. initial can only execute once.

B. The difference between initial and always is that the former cannot be synthesized, while the latter can be synthesized.

C. always executes continuously in a loop.

D. initial and always cannot execute simultaneously.

(b)Assume [wire [7:0] data = 8'b11010010;] then data[3:0] = _________. (4 marks)

(c)Assume that the width of the input signal X is 10, the width of the input signal A is 5, and the width of the input signal B 14. In order to achieve the function of Y = X * A + B without losing accuracy, what is the minimum width of the output signal Y? Assuming all signals are signed. _________. (4 marks)

(d) Given the below code, draw the waveform of all the signals. (5 marks)

  - module testbench();

  - reg clk, rst, q0, q1, q2, q3;

  - always@(posedge clk) begin

  - if (rst) begin

  - q0 <= 0;

  - q1 <= 0;

  - q2 <= 0;

  - q3 <= 0;

  - end

  - else begin

  - q1 <= q0;

  - q2 = ~q1;

  - q3 <= q2 + 1;

  - end

  - end

  - always#5 begin

  - clk <= ~clk;

  - end

  - initial begin

  - rst = 1;

  - clk = 0;

  - #10;

  - rst = 0;

  - q0 = 1;

  - #50;

  - $finish;

  - end

  - endmodule

(e) Draw the block diagram of below two modules using logic gates and registers. (8 marks)

- module M1(

- input A, B, C, D,

- output E

- );

- assign E = !((A^B)&(~C&D));

- endmodule

- module M2(

- input clk, A, B, C, D,

- output reg E

- );

- reg F, G;

- always@(posedge clk) begin

- F <= A | B;

- G <= B & C;

- E = ~A ^ (F | G)

- end

- endmodule

Q.2

- Design an automatic beverage vending machine that sells drinks for 10 cents. The machine accepts two coin types: 5 cents (in = 1) and 10 cents (in = 2). When the total inserted amount reaches or exceeds 10 cents, the output signal out should assert immediately to dispense the beverage. The machine must also handle change: whenever change is required, the corresponding change output should assert immediately. Draw the corresponding finite state machine (FSM), and implement it in Verilog with an asynchronous reset that returns the FSM to its initial state. (10 marks)

- module top_module(

- input clk,

- input [1:0] in,

- input areset,

- output out,

- output change); //

- // State transition logic

- // State flip-flops with asynchronous reset

- endmodule

- LUT, or Lookup Table, is a fundamental building block in digital design and is commonly used in FPGAs. A LUT is a small, configurable memory unit that stores data and is used to implement combinational logic functions.

- How many different 3 input functions can a 3 input LUT implement? Why? (5 marks)

- Using nothing other than 3-LUTs, demonstrate how you would construct a 6-LUT. Label your inputs x0, x1...., x5, and output as y.(5 marks)

Q.3

Given the following Verilog code:

- Draw the schematic of the hardware that will be synthesized from the Verilog code below. (8 marks)

- How many timing paths are in the schematic generated from the code? What are they?    (5 marks)

- module Top(

- input CLK,

- input RST,

- input in,

- output out

- );

- wire A;

- reg B;

- reg C;

- wire D;

- reg E;

- wire F;

- assign A = in;

- always @(posedge CLK or posedge RST)

- begin

- if(RST == 1'b1)

- B <= 1'b0;

- else

- B <= A ;

- end

- assign D = ~ B & C;

- assign F = D | E;

- always @ (*)

- begin

- E = B ^ C;

- end

- always @(posedge CLK or posedge RST)

- begin

- if(RST == 1'b1)

- C <= 1'b0;

- else

- C <= F ;

- end

- assign out = C;

- endmodule

Q.4

(a) In a generic 3x3 Image Filter design, we use a LineBuffer and a window array to process the stream. We need to implement a "Sliding Window" operation inside the column loop.

Context:

- LineBuffer[3][WIDTH] stores the necessary image rows. Assume row 0, 1, 2 of the LineBuffer correspond to the top, middle, and bottom rows of the filter window respectively.

- window[3][3] is the local register array.

- j is the current column index.

Complete the code to:

- Shift the data within the window to the left (prepare for new data).

- Load the new column data from the LineBuffer into the rightmost column of the window.

(6 marks)

- PIC_COLS: for(int j = 0; j < cols; j++) {

- window[0][0] = window[0][1];

- ___________________________;

- ___________________________;

- ___________________________;

- ___________________________;

- ___________________________;

- ___________________________;

- ___________________________;

- ___________________________;

- // Execute the Kernel (e.g., Average, Sobel, Gaussian)

- PIXEL result = apply_filter(window);

- dst[j] = result;

- }

(b) To achieve II=1, we apply #pragma HLS ARRAY_PARTITION variable=window complete.

- Explain why this is necessary for the window array. (3 marks)

- If we remove this pragma, what hardware resource (RAM or Registers) will the tool likely use for window, and how will it affect the performance? (3 marks)

Q.5

(a) There are three AXI interfaces in Vitis HLS, list their names and explain their differences. (6 marks)

(b) Given the below code

- void foo(int A[32], int B[32], int C[32]) {

- #pragma HLS interface m_axi port=A bundle=D1

- #pragma HLS interface m_axi port=B bundle=D1

- #pragma HLS interface m_axi port=C bundle=D2

- for (int i=1; i<32; i++) {

- B[i] = A[i] + A[i-1] + C[i] + C[i-1];

- }

- }

- Use PIPELINE pragma to optimize this function, where should you insert it? (2 marks)

- What is the minimum Initial Interval (II) it can achieve after applying PIPELINE? Explain why. (4 marks)

- In order to achieve II=1, how you can modify the code? Describe in words or code.  (8 marks)

(c) Given the below code, using array partition pragmas to meet the target II. (6 marks) You can write down the pragmas directly or describe in words. Assume all the arrays are implemented by single port BRAMs, what is the minimum number of BRAM required?  (4 marks)

- int A[32][32];

- int B[32][32];

- int C[32];

- for (int i = 0; i < 32; i++) {

- for (int k = 0; k < 32; k++) {

- #pragma HLS PIPELINE II=1

- int result = 0;

- for (int j = 0; j < 32; j++) {

- result += A[i][k] * B[k][j] + A[31-i][k];

- }

- C[k] = result;

- }

- }

END OF PAPER